# Lab # 3

Objective: Verify the operation of BCD to Decimal Converter.

## Equipment:

  - 4511 BCD to 7-Segment Decoder IC

  - 74LS47 BCD to 7-Segment Decoder IC

  - 7-Segment Display

  - Breadboard

  - Connecting wires

  - LEDs

  - Resistors (220Ω)

  - Power supply (5V)

  - Switches

  - Multimeter/oscilloscope

## Theory:

BCD (Binary-Coded Decimal) to Decimal Converter translates a BCD input into its decimal representation on a 7-segment display. The BCD input represents decimal digits in binary format.

## Procedure:

- Connect the power supply to the breadboard.

- Place the 4511 IC (BCD to 7-Segment Decoder) or 74LS47 IC (BCD to 7-Segment Decoder) on the breadboard.

- Connect switches to the BCD inputs of the IC.

- Connect the output of the IC to a 7-segment display.

- Set the BCD input values and observe the corresponding decimal output on the display.

- Record the observed outputs for different BCD inputs.

## Observation Table:

| BCD Input | Decimal Output (7-Segment Display) |
| --- | --- |
| 0000 | 0 |
| 0001 | 1 |
| 0010 | 2 |
| 0011 | 3 |
| 0100 | 4 |
| 0101 | 5 |
| 0110 | 6 |

| BCD Input | Decimal Output (7-Segment Display) |
| --- | --- |
| 0111 | 7 |
| 1000 | 8 |
| 1001 | 9 |
| 1010 | Invalid |
| ... | Invalid |

Results:

The BCD to Decimal Converter correctly translates BCD inputs to decimal numbers on the 7-segment display.

## Conclusion:

The BCD to Decimal Converter works as expected, accurately displaying decimal digits corresponding to the BCD inputs.

## Safety Precautions:

- Verify the power supply is set to 5V to prevent damage.

- Handle the 7-segment display and ICs carefully to avoid static damage.

Circuit Diagram:

POST LAB:

- How does the BCD input correlate with the decimal output on the 7-segment display?

- What happens if an invalid BCD input (e.g., 1010) is provided?

- Describe the role of the BCD to 7-Segment Decoder IC in this circuit.