# Lab # 4

Objective: Verify the operation of Decimal to BCD Converter.

## Equipment:

  - 4516 Decimal to BCD Converter IC

  - 7-Segment Display

  - Breadboard

  - Connecting wires

  - LEDs

  - Resistors (220Ω)

  - Power supply (5V)

  - Switches

  - Multimeter/oscilloscope

## Theory:

Decimal to BCD Converter translates decimal numbers into their BCD representation. The output can be used to drive BCD displays or other digital systems requiring binary-coded decimal inputs.

## Procedure:

- Connect the power supply to the breadboard.

- Place the 4516 IC (Decimal to BCD Converter) on the breadboard.

- Connect a decimal input to the IC using switches.

- Connect the output of the IC to a 7-segment display or LED indicators.

- Set the decimal input values and observe the corresponding BCD output.

- Record the observed outputs for different decimal inputs.

## Observation Table:

| Decimal Input | BCD Output |
| --- | --- |
| 0 | 0000 |
| 1 | 0001 |
| 2 | 0010 |
| 3 | 0011 |
| 4 | 0100 |
| 5 | 0101 |
| 6 | 0110 |
| 7 | 0111 |

| Decimal Input | BCD Output |
| --- | --- |
| 8 | 1000 |
| 9 | 1001 |
| 10 | Invalid |
| ... | Invalid |

Results:

The Decimal to BCD Converter accurately converts decimal numbers to their BCD representation, displayed on LEDs or a 7-segment display.

## Conclusion:

The Decimal to BCD Converter functions correctly, providing accurate BCD outputs for valid decimal inputs.

## Safety Precautions:

- Ensure the power supply is correctly set to 5V.

- Handle ICs an

- d displays carefully to avoid damage from static or improper connections.

Circuit Diagram:

POST LAB:

- How does the Decimal to BCD Converter handle inputs greater than 9?

- Describe the function of the Decimal to BCD Converter IC in this circuit.

- What is the significance of BCD output in digital systems?