# Lab # 2

Objective: Assemble Full Adder and verify its operation.

## Equipment:

  - 7408 IC (AND gate)

  - 7432 IC (OR gate)

  - 7486 IC (XOR gate)

  - Breadboard

  - Connecting wires

  - LEDs

  - Resistors (220Ω)

  - Power supply (5V)

  - Switches

  - Multimeter/oscilloscope

## Theory:

A Full Adder adds three binary digits (including a carry-in) and provides two outputs: Sum and Carry. It consists of two XOR gates, two AND gates, and one OR gate.

## Procedure:

- Connect the power supply to the breadboard.

- Place the 7486 IC (XOR gate), 7408 IC (AND gate), and 7432 IC (OR gate) on the breadboard.

- Connect the inputs A and B to switches and the Carry-In input.

- Connect the outputs of the XOR gates and AND gates to LEDs to indicate Sum and Carry outputs.

- Connect the output of the OR gate (for the final carry) to an LED.

- Test all combinations of input values (0, 1) and Carry-In values (0, 1).

- Observe and record the outputs.

## Observation Table:

| Input A | Input B | Carry-In | Sum (Output) | Carry (Output) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |

| Input A | Input B | Carry-In | Sum (Output) | Carry (Output) |
| --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 |

Result:

The Full Adder circuit produces the correct Sum and Carry outputs for all input combinations and carry-in values.

## Conclusion:

The Full Adder functions as intended, successfully adding two binary numbers with a carry-in and providing the correct sum and carry outputs.

## Safety Precautions:

- Verify the power supply is set to 5V to prevent damage to the ICs.

- Use proper grounding to avoid short circuits.

Circuit Diagram:

POST LAB:

- What role do the XOR gates play in the Full Adder circuit?

- How does the OR gate contribute to the final Carry output?

- Explain the difference between the Full Adder and the Half Adder in terms of input and functionality.