# Lab # 1

Objective: Assemble Half Adder and verify its operation.

## Equipment:

- 7408 IC (AND gate)

- 7432 IC (OR gate)

- 7404 IC (NOT gate)

- Breadboard

- Connecting wires

- LEDs

- Resistors (220Ω)

- Power supply (5V)

- Switches

- Multi meter/oscilloscope

## Theory:

A Half Adder is a digital circuit that adds two binary digits and provides two outputs: Sum and Carry. It uses an XOR gate to compute the sum and an AND gate to compute the carry.

## Procedure:

- Connect the power supply to the breadboard.

- Place the 7408 IC (AND gate) and 7432 IC (OR gate) on the breadboard.

- Connect the inputs of the XOR gate (from 7408 IC) to two switches.

- Connect the output of the XOR gate to an LED (indicating the Sum output).

- Connect the inputs of the AND gate (from 7408 IC) to the same two switches.

- Connect the output of the AND gate to another LED (indicating the Carry output).

- Test all combinations of input values (00, 01, 10, 11) using the switches.

- Observe and record the outputs of Sum and Carry.

## Observation Table:

| Input A | Input B | Sum (Output) | Carry (Output) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

Results:

The Half Adder successfully provides the correct Sum and Carry outputs based on the binary inputs.

## Conclusion:

The Half Adder circuit works as expected, correctly computing the sum and carry for all possible input combinations.

## Safety Precautions:

- Ensure the power supply is correctly set to 5V to avoid damage to the ICs.

- Handle ICs with care to avoid static damage.

Circuit Diagram:

POST LAB:

- What is the purpose of the XOR gate in the Half Adder circuit?

- How does the AND gate contribute to the operation of the Half Adder?

- Explain why the Carry output is 1 only when both inputs are 1.